| TRAINING OCCUPATION: ELECTRICAL INSTALLATION & MAINTENANCE (SINGLE PHASE & THREE PHASE) (NOSS: EE-320-2/3: 2012) |  |  |
| --- | --- | --- |
| CWP 3: ELECTRICAL MOTOR AND MOTOR CONTROL INSTALLATION |  |  |
| CU 3.1: SINGLE PHASE MOTOR & MOTOR CONTROL INSTALLATION |  | CODE: |
| Work Activity 1 | Identify single phase motor & motor control installation specifications and procedures | Duration: 12 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - know related knowledge for installation and motor function system including various type motor using in market for single phase system - List type of PPE for single phase motor installation and connection. - Explain the function for each type of single phase motors, such as: - Capacitor start - It will use domestic water pump, air cond, weld pump, booster pump based on customer requirements - Capacitor run - Exhaust fan, chimney, blower system, car auto spray system - Universal motor AC/DC - Commonly use for auto gate, roller shutter, retractable awning, and oil industry (refinery) - List single phase motor parts, such as: - Bearing, - Brush - Shaft - Rotor - Stator - Explain application of single phase motors control according to type below:- - Direct Online (DOL) - used for air cond , water pump, any motor more than ½ hp - 3hp - Forward Reverse - Used at kitchen wares like pulp milling machine, dish washer - Cam starter - used for small equipment were by rating of amps are low for forward and reverse without any contactor especially use double capacitor at grinding machine, blender, etc - Explain single phase motor control components, such as: - Magnetic contactor - push button - overload relay & 8 pin relay A/C - connector - terminal block - MCB & MCCB - Timer Delay Relay (TDR) - auxiliary contact, latching relay - pilot lamp or indicator lamp - HRC Fuse - Describe Electrical Act & Regulation and Standard and regulation for single phase installation, such as: - Electrical Regulation 1994 - MS IEC 60364 - MS 1936: 2006 Guide to MS IEC 60364 - MS 1979: 2007 Electrical Installation of Buildings – Code of practice - Based on latest instruction by regulatory body - Occupational Safety & Health Act 514 (OSHA) 1994 requirement for single phase motor installation. - List authorisation requirement to access site for single phase motor and motor control installation, such as: - Green card, - Safety passport. |  |  |
| Work Activity 2 | Prepare single phase motor and motor control installation, maintenance, tools, equipment | Duration: 16 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to do draw and choose suitable drawing for relevant task - List type of motor circuit and motor control diagram, such as: - Single line drawing - block drawing - blue print drawing - shop drawing - layout drawing - as built drawing - Describe procedure to draw main circuit and control circuit (Shop drawing) according to motor circuit and motor control diagram. - Explain procedure to produce as built drawing for single phase motor installation. - Describe purpose of as built drawing for single phase motor installation. - List location of motor & motor control installation devices including: - Platform structure - Underwater - Surface - Weather proof - Protection devices - Sensing devices - List type of maintenance for Single phase motor & motor control, such as: - Periodically check - Preventive maintenance - Corrective maintenance - List type of installation tools, equipment and materials for Single phase motor & motor control, such as: - Hand tools - Power tools - Instrument - Explain purpose of single phase motor & motor control testing instrument, such as: - LCR meter - To check micro farad uf capacitor - Multi-meter - Measure AC/DC, ohms and continuity - Insulation tester - Insulation test for phase to neutral and phase to earth to confirm insulation of conductor are good condition and resistant - Tacho-meter |  |  |
| Work Activity 3 | Carry out single phase motor installation | Duration: 12 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - Elaborate circuit diagram for single phase motor installation. - List single phase motor symbol. - Explain single phase motor sizing & rating. - Explain function of single phase motor control panel accessories. - Explain single phase motor starter functionality of: - Main circuit - Control circuit. - Explain single phase motor installation procedure. |  |  |
| Work Activity 4 | Carry out single phase motor control termination and connection | Duration: 8 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - Explain single phase motor and motor control termination & connection procedure. - List types of single phase motor and motor control connection circuit, such as: - Direct On Line (DOL), - forward/reverse, - Soft starter. - List tools of single phase motor and motor control for connection and fitting works. - Describe operation of single phase motor and motor control hand and power tools. - Describe single phase motor and motor control marking and labelling procedure. |  |  |
| Work Activity 5 | Inspect single phase motor and motor control functionality | Duration: 12 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - Describe application of protection devices for single phase motor and motor control. - Explain single phase motor and motor control functionality inspection methods, such as: - Visual inspection, - Physical Check (sound, connection). - Explain testing procedure of single phase motor and motor control, such as: - Insulation - Continuity - Polarity - Earthing - Load test (Speed and current) - Explain functionality of single phase motor and motor control according to specification/ operation manual. |  |  |
| Work Activity 6 | Record single phase motor and motor control installation. | Duration: 4 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - Explain single phase wiring motor and motor control recording procedure. - Explain single phase motor and motor control record testing format. - Explain single phase motor and motor control report submission procedure. |  |  |
| TOTAL (CU 5.1: SINGLE PHASE MOTOR & MOTOR CONTROL INSTALLATION) |  | Duration: 64 Hours |