| LEARN AND WORK ASIGNMENT (LWA) |  |  |  |
| --- | --- | --- | --- |
| NOSS (CODE NOSS) | MOTORCYCLE SERVICING AND MAINTENANCE (TP-118-1;2012) |  |  |
| Competency Unit Title (CU CODE) | PERFORMANCE TYRE AND WHEEL SERVICING (ETM-XXX-05) | LEVEL | 2 |
| Competency Unit Descriptor | Performance tyre and wheel servicing competency is a set of skills to carry out performance tyre and wheel servicing including inspection on performance tyre and wheel defect, replacement of performance tyres and wheels components and executing performance tyre and wheel performance regulation according to motorcycle service’s manual. The personnel whom is competent in performance tyre and wheel servicing must be able to prepare performance tyre and wheel servicing requirements, perform test free wheel rotation, perform front and rear wheel bearing servicing, perform tube and tubeless type performance tyres servicing, perform performance tyre and wheel servicing performance check and record performance tyre and wheel servicing activities |  |  |
| Candidate Name |  |  |  |
| Candidate I/C Number |  |  |  |
| Company’s Name |  |  |  |

CU WORK ACTIVITY STATEMENT:

PERFORM PERFORMANCE TYRE AND WHEEL SERVICING ACTIVITIES

DURATION:  85   HOURS

- SETTING GOAL :

You are required to perform performance tyre and wheel servicing activities based on performance criteria below:

  - Performance tyre and wheel servicing tools and equipment prepared on work area

  - Performance tyre and wheel servicing manual referred

  - Performance tyre defect identified

  - Rim defect identified

  - Bearing defect identified

  - Front and rear wheel bearing serviced according to motorcycle service’s manual

  - Types of performance tyre servicing identified

  - Performance tyre tube replaced according to motorcycle service’s manual

  - Types of performance tyre servicing identified

  - Performance tyre replaced according to motorcycle service’s manual

  - Wheel spokes servicing tools and equipment prepared on work area

  - Wheel spokes servicing manual referred

  - Wheel spokes serviced according to service’s manual

  - Cast wheel servicing tools and equipment prepared on work area

  - Cast wheel servicing manual referred

  - Cast wheel serviced according to service’s manual

  - Performance tyre and wheel balancing tools and equipment prepared on work area

  - Performance tyre and wheel balancing manual referred

  - Performance tyre and wheel balancing serviced according to service’s manual

  - Performance tyre and wheel performance checked against motorcycle service’s manual

  - Motorcycle test ride executed

  - Service checklist updated

  - Service checklist submitted to   superior

B. PLANNING

You are required to plan activities to achieve listed setting goal of performing performance tyre and wheel servicing activities by using resources listed below:

  - Identify  tools, materials and equipment for performing performance tyre and wheel servicing activities according to listed below:

| ITEMS | RATIO (TEM : Trainees) |
| --- | --- |
| - Hand tools - Jack / stand - Performance tyre pressure gauge - Performance tyre repair kit - Performance tyre lever set - Wheel assembly tube type - Wheel assembly tubeless type - Tube - Rim tape/band rim - Performance tyre grease - Performance tyre depth gauge - Trueing stand - Dial gauge c/w stand - Spoke wrench - Performance tyre balancing machine - Performance tyre changing machine - Air gun - Balancing weight - Performance tyre valve assembly - Air compressor - Personal Protective Equipment (PPE) - Service manual | 1 :1 1 ; 1 1 : 5 1 : 5 1 : 5 1 : 1 1 : 1 As per required 1 : 1 1 : 5 1 : 5 1 : 5 1 : 5 1 : 5 1 : 10 1 : 10 1 : 10 1 : 10 As required 1 : 10 1 : 1 1 : 10 1 : 1 1 : 5 |

  - You may refer to references below as a guidance to perform this activity.

REFERENCES

- Alberta. (2005). Alberta Advanced Education and Technology Apprenticeship and Industry Training. Motorcycle Mechanic Apprenticeship Course Outline. Government of Alberta. ISBN 077854317X - Dave Tharp. (1991)Trelland’s Motorcycle Reference Guide – Vol. 1. ISBN 1-895542-00-6. Trelland Industries Ltd - de Rome, L., Stanford, G. & Wood, B. (2007), Positioned for Safety 2010: A Motorcycle Safety Strategic Plan 2007-2010, Motorcycle Council of NSW, Inc., - Gibson T. & E. Benetatos (2000), Motorcycles and crash barriers prepared by Human Impact Engineering for the Barriers to Safety campaign, NSW Motorcycle Council, Sydney - Hurt, H.H. Jr., Ouellet, J.V. & Thom, D.R. (1981), Motorcycle Accident Cause Factors and Identification of Countermeasures, Final Report to National Highway Traffic Safety Administration, US Department of Transportation, PB 81-206443, 81-206450

  - List out any references related  when performing this activity such as:

    - Standard Operating Manual (SOP)/ Service manual / Manufacturer’s manual

    - Service job sheet

    - Owner’s manual

    - Etc.

- DESION MAKING

You are required to get coach approval before performing performance tyre and wheel servicing activities.

- EXECUTE & MONITORING

You are required to carry out  performance tyre and wheel servicing activities according to steps below:

  - Prepare performance tyre and wheel servicing requirements

    - Interpret performance tyre and wheel servicing according to service manual.

    - Determine performance tyre and wheel servicing activities

- Refer to service job sheet for motorcycle’s model and type by SA (Service Advisor)

    - Select performance tyre and wheel servicing tools and equipment

  - Perform test free wheel rotation

    - Determine procedure to carry out  test free wheel rotation

    - Test free wheel rotation according to procedure and service manual

    - Determine type of defect on performance tyre and wheel by visual and sound inspection to identify :

- Bearing

- Wobbling

- Performance tyre balancing(static)

  - Dismantle wheel from motorcycle

  - Perform front and rear wheel bearing servicing

    - Determine types and functions  of front and rear wheel bearing

    - Replace front and rear wheel bearing and components below  according to service’s manual:

- Bearing,

- Dust cap,

- Oil seal,

- Collar bush,

- Damper hub,

- Rim and

- Etc

  - Perform  performance tyre servicing

    - Determine types of  performance tyre

- Refer to service job sheet and service manual

    - Check  performance tyre specification according to performance tyre marking such as:

- Production date/manufacturer date (refer to performance tyre marking)

- Size of performance tyre

- Performance tyre rotation

- Performance tyre maximum pressure

- Performance tyre type

- Performance tyre load index

- Performance tyre wear indicator(TWI)

- Performance tyre moulded line

- Number of ply

- Etc.

  - Dismantle performance tyre from rim

  - Replace performance tyre according to service manual

  - Perform wheel spokes servicing

    - Determine types and functions of wheel spokes components

    - Service wheel spokes according to procedure and service manual

- Replace if spokes broken, rusty, and etc.

- Adjust if spokes is  wobbling

  - Perform  cast wheel servicing

    - Determine types and functions of cast wheel pattern

    - Replace/ service cast wheel according to procedure and service manual

  - Assemble performance tyre to rim motorcycle

    - Check performance tyre pressure according specification and leakage

  - Perform wheel balancing

    - Determine types and functions of wheel balancing

    - Operate performance tyre balancing machine according to operation manual

    - Carry out wheel balancing according to procedure and service manual

- Check if the wheel is wobbling

- Add weight balancer if required.

  - Assemble wheel to motorcycle.

  - Perform wheel servicing functionality

    - Check wheel and brake functionality according to service manual

    - Inform SA that motorcycle ready for test ride

    - Carry out test  ride on serviced motorcycle

  - Record final drive servicing activities

  - Submit record to SA

  - Carry out housekeeping activities such as:-

    - 5’S

    - Etc.

  - Comply with attitude, safety and environment listed below when performing this activity

| Attitude | - Peruse the service’s specification and maintenance manual - Adhere to workshop safety practice - Meticulous in preparing report |
| --- | --- |
| Safety | - Wear appropriate PPE - Handle wiring tools, equipment and testing tools with care - Ensure tools and equipment in good working condition |
| Environment | - |

  - Apply core abilities listed below when performing this activity

| Social Skills | Core Abilities |
| --- | --- |
| Communication skills | 02.03 Communicate clearly |
| Conceptual skills | 01.01 Identify and gather information 02.01 Interpret and follow manuals, instructions and SOP’s |
| Interpersonal skills | 03.06 Respond appropriately to people and situations |
| Multitasking and prioritizing | 01.02 Document information, procedures or processes 02.04 Prepare brief reports and checklists using standard form |
| Self-discipline | 03.03 Accept responsibility for own work and work area 03.02 Demonstrate integrity and apply ethical practices |
| Teamwork | 03.01 Apply cultural requirements for the workplace |
| Learning skill | 06.01 Understand systems |
| Leadership skill | 03.06 Respond appropriately to people and situations 03.03 Accept responsibility for own work and work area |

- EVALUATING

You are required to evaluate performance tyre and wheel servicing activities using checklist below.

| A | ASSESSMENT CRITERIA (60%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Performance tyre and wheel servicing tools, equipment’s, and workplace are prepared |  |  |  |  |  |  |  |  |  |  |
| 2 | Free wheel tested according to procedure and service’s Manual |  |  |  |  |  |  |  |  |  |  |
| 3 | Front and rear wheel bearing checked according to procedure and service’s Manual |  |  |  |  |  |  |  |  |  |  |
| 4 | Performance tyre changed according to procedure and service manual. |  |  |  |  |  |  |  |  |  |  |
| 5. | Replace wheel spokes/Cast wheel according to procedure and service manual |  |  |  |  |  |  |  |  |  |  |
| 6. | Perform performance tyre and wheel balancing according to procedure and service manual. |  |  |  |  |  |  |  |  |  |  |
| 7. | Update activities are recorded according to correct format. |  |  |  |  |  |  |  |  |  |  |
|  | SUBTOTAL | A1 |  |  |  |  | A2 |  |  |  |  |
|  | FULL MARKS | 49 |  |  |  |  | 49 |  |  |  |  |
| B | ATTITUDE/SAFETY/ ENVIRONMENT (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Attitude |  |  |  |  |  |  |  |  |  |  |
| 2. | Safety |  |  |  |  |  |  |  |  |  |  |
| 3 | Environmental |  |  |  |  |  |  |  |  |  |  |
|  |  | B1 |  |  |  |  | B2 |  |  |  |  |
|  |  | 21 |  |  |  |  | 21 |  |  |  |  |
| C | EMPLOYABILITY SKILLS (SOCIAL SKILLS) (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Communication skills |  |  |  |  |  |  |  |  |  |  |
| 2 | Conceptual skills |  |  |  |  |  |  |  |  |  |  |
| 3 | Interpersonal skills |  |  |  |  |  |  |  |  |  |  |
| 4 | Multitasking and prioritizing |  |  |  |  |  |  |  |  |  |  |
| 5 | Self-discipline |  |  |  |  |  |  |  |  |  |  |
| 6 | Teamwork |  |  |  |  |  |  |  |  |  |  |
| 7 | Learning skill |  |  |  |  |  |  |  |  |  |  |
| 8 | Leadership skill |  |  |  |  |  |  |  |  |  |  |
|  |  | C1 |  |  |  |  | C2 |  |  |  |  |
|  | FULL MARKS | 56 |  |  |  |  | 56 |  |  |  |  |

CALCULATION TABLE

|  | MARKS GIVEN BY APPRENTICE | MARKS GIVEN BY COACH | WEIGHTED MARKS GIVEN BY APPRENTICE | WEIGHTED MARKS GIVEN BY COACH |
| --- | --- | --- | --- | --- |
| ASSESSMENT CRITERIA | A1 | A2 | A1 / 49 X 60 | A2 /49 X 60 |
| ATTITUDE,SAFETY & ENVIRONMENT | B1 | B2 | B1 / 21X 20 | B2 / 21 X 20 |
| EMPLOYABILITY SKILLS (SOCIAL SKILLS) | C1 | C2 | C1 / 56 X 20 | C2 / 56 X 20 |
| Total |  |  | X | Y |
| Ratio of Percentage (Apprentice: Coach) |  |  | P% | Q% |
| Grand Total |  |  | (P/100 x X) + (Q/100 x Y) |  |

COMMENTS/ RECOMMENDATIONS BY COACH

_________________________			  ______________________

COACH: 					              APPRENTICE:

DATE:							  DATE: