| MOTORCYCLE SERVICING AND MAINTENANCE |  |  |
| --- | --- | --- |
| Competency Unit (CU) : Anti-lock brake system servicing |  | Code : |
| Work Activity 1: Prepare anti-lock brake system requirements |  | Duration : 3 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - Understand related manual content on anti-lock brake system such as: - Service manual: Servicing schedule, Wiring diagram and etc. - Operating manual : Method of servicing, Tools and equipment - Part catalogue: Part number, Full diagram, etc - Etc. - List out tools and equipments used for anti-lock brake system such as: - Air bleed vacuum, - Hand tools, - Special tools, - Service manual, - Multi meter, - Laptop / diagnostic equipment, and - Etc - Explain function and operation of anti-lock brake system such as: - Brake stability, - Anti skid, and - Etc - Describe types of anti-lock brake system such as: : - Integrated - Non-integrated - Describe anti-lock brake system components involved in inspection activities such as: - Brake malfunction - Sensor short circuit - Indicator lamp(abs) always on at meter panel while riding - Inspect wiring connector - Inspect speed sensor for bend - Inspect chipped sensor for rotor teeth/tooth wheel abs sensor (air gapping) - Inspect brake calliper assembly, brake pad wear and tear - Brake disc for cracking, pitting or excessive rust |  |  |
| Work Activity 2 : Perform brake control module servicing/inspection |  | Duration : 10 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - Understand function of brake control module such as: - Control hydraulic pressure in brake system, and - Etc, - Explain procedure of brake control module servicing / inspection according service manual - Describe types and functions of brake control module component such as: - Sensor, - Wire connector, - Solenoid valve assembly, - Meter panel assembly, - Brake calliper assembly, - Brake pump assembly, - Relay assembly, - Brake hose, - ECU, - Brake disc - Traction control - List out tools and equipment used for brake control module servicing/inspection such as: - Hand tools - Special tools - Service manual - Multi meter - Computer unit - Wiring harness - Etc. |  |  |
| Work Activity 3 : Perform solenoid valve assembly servicing/inspection |  | Duration : 10 Hours |
| Learning Objectives At the end of learning session the apprentice will be able to: - Understand function of solenoid valve assembly such as: - To control brake fluid pressure to the wheel when signal receive from ABS-ECU, - Etc. - Describe procedure of solenoid valve assembly servicing/ inspection - Describe types and functions of solenoid valve assembly components such as: - Plunger, - Diaphragm, - Spring, - Housing, and - Etc - List out tools and equipment used for anti-lock brake system servicing/inspection such as: - Hand tools - Special tools - Service manual - Brake fluid - Torque wrench - Vacuum pump - Part tray - Waste cloth/rag - Etc. |  |  |
| Work Activity 4 : Perform speed sensor and wiring servicing/inspection | Duration : 10 Hours |  |
| Learning Objectives At the end of learning session the apprentice will be able to: - Understand the function of speed sensor such as: - Measure rotation of wheel , - Send information to ECU, - Indicating light, - Etc. - Understand procedure for servicing/ inspect ion of speed sensor and wiring according to service manual - Describe types and functions of speed sensor and wiring component such as: - Sensor - Wire harness - Etc. |  |  |
| Work Activity 5 : Perform anti-lock brake system functionality check | Duration : 10 Hours |  |
| Learning Objectives At the end of learning session the apprentice will be able to: - Understand procedure to carry out motorcycle test ride according to service manual such as: - Safety riding practice, - Pre- ride test/Pre-start procedure, and - Etc. - Explain procedure to check anti-lock brake system functionality according to service manual |  |  |
| Work Activity 6 : Record anti-lock brake system servicing/inspection activities | Duration : 3 Hours |  |
| Learning Objectives At the end of learning session the apprentice will be able to: - List types of checklist to record anti-lock brake system servicing/inspection activities such as: - Service checklist - Job card - Vehicle repair order - Inventory checklist - Etc - Understand procedure to record anti-lock brake system servicing/inspection activities according to company’s SOP (Standard Operating Procedure) |  |  |
| TOTAL | 46 Hours |  |