| LEARN AND WORK ASIGNMENT (LWA) |  |  |  |
| --- | --- | --- | --- |
| NOSS (CODE NOSS) | MOTORCYCLE SERVICING AND MAINTENANCE (TP-118-1; 2012) |  |  |
| Competency Unit Title (CU CODE) | MOTORCYCLE SCHEDULED MAINTENANCE (TP-118-1-C01) | LEVEL | 1 |
| Competency Unit Descriptor | Motorcycle scheduled maintenance is a set competency of skills to carry out preventive maintenance that has been recognized by manufacturer’s manual requirements. The personnel whom are competent in the motorcycle schedule maintenance must be able to prepare scheduled maintenance requirements, perform engine oil, oil filter, air filter, final drive gear oil, spark plugs and coolant changing. Also able to perform the valve clearance adjustment, motorcycle functionality check and record schedule maintenance activities. Motorcycle schedule maintenance is to ensure that the motorcycle can maintain its performance, safe and a longer life expectancy. |  |  |
| Candidate Name |  |  |  |
| Candidate I/C Number |  |  |  |
| Company’s Name |  |  |  |

CU WORK ACTIVITY STATEMENT:

PERFORM MOTORCYCLE SCHEDULED MAINTENANCE ACTIVITIES

DURATION:   44 HOURS

- SETTING GOAL *:

You are required to perform motorcycle scheduled maintenance activities based on performance criteria below:

  - Maintenance tools and equipment prepared on work area

  - Manufacturer’s schedule maintenance manual determined

  - Select engine oil and oil filter correctly

  - Engine oil changed according to procedure

  - Oil filter change according to manufacturer’s manual

  - Select air filter correctly

  - Air filter changed according to procedure and manufacturer’s manual

  - Select final drive gear oil correctly

  - Final drive gear oil changed according to procedure and manufacturer’s manual

  - Used oil disposes correctly according to procedure

  - Select spark plug correctly

  - Spark plugs changed according to procedure and manufacturer’s manual

  - Select coolant type correctly

  - Change coolant according to procedure and manufacturer’s manual

  - Valve clearance adjusted according to the motorcycle manufacturer’s manual

  - Brake system performance checked

  - Schedule maintenance checklist updated

  - Schedule maintenance checklist submitted to superior

- PLANNING

You are required to plan activities to achieve listed setting goals of performing motorcycle scheduled maintenance activities by using the resources listed below:

  - Identify  tools, materials and equipment for performing motorcycle scheduled maintenance according to listed below:

| ITEMS | RATIO (TEM : Trainees) |
| --- | --- |
| - Engine oil - Semi synthetic - Full systemic - Mineral based - Oil filter - Cartridge/element - Spin on type - Mesh - Pan/sump - Drain plug - Hand tools | As per required 1 : 1 1 : 1 1 : 1 1 : 1 |
| - Funnel with narrow neck - Container - Service manual - Final drive gear oil - Semi synthetic gear oil - Fully synthetic gear oil - Mineral based gear oil - Air filter - Paper - Foam - Cotton gauze - Solvent - Air duster gun - Air compressor - Coolant - Personnel Protective Equipment (PPE) - Measuring beaker - Special tools - Torque wrench - Oil filter wrenches - Valve adjustment tool - Feeler gauge | 1 : 1 1 : 1 1 : 1 As per required As per required: As per required 1 : 10 1 : 10 As per required 1 : 1 1 : 10 - : 10 |

  - Refer to the references below as a guide to perform this activity.

REFERENCES

- Mark Zimmerman. (2004).The essential guide to motorcycle maintenance: Tips & Techniques to Keep Your Motorcycle in Top Condition. ISBN1884313418, 9781884313417. Whitehorse Press Series. - Dave Tharp. (1991)Trelland’s Motorcycle Reference Guide – Vol. 1. ISBN 1-895542-00-6. Trelland Industries Ltd - de Rome, L., Stanford, G. & Wood, B. (2007), Positioned for Safety 2010: A Motorcycle Safety Strategic Plan 2007-2010, Motorcycle Council of NSW, Inc., - Gibson T. & E. Benetatos (2000), Motorcycles and crash barriers prepared by Human Impact Engineering for the Barriers to Safety campaign, NSW Motorcycle Council, Sydney - Hurt, H.H. Jr., Ouellet, J.V. & Thom, D.R. (1981), Motorcycle Accident Cause Factors and Identification of Countermeasures, Final Report to National Highway Traffic Safety Administration, US Department of Transportation, PB 81-206443, 81-206450

  - List out any references when performing this activity such as:

    - Service manual / Manufacturer’s manual/ Workshop manual

    - Service job sheet

    - Etc.

- DESION MAKING

You are required to get coach approval before performing motorcycle scheduled maintenance activities.

- EXECUTE & MONITORING

You are required to perform motorcycle scheduled maintenance activities according to the steps below:

  - Prepare scheduled maintenance requirements

    - Interpret manufacturer’s scheduled maintenance manual

    - Determine scheduled maintenance servicing activities

- What are the model, mileage/duration of the motorcycle to service? You can refer to job sheet given by your SA (Service Advisor)

  - Prepare tools, equipment and materials before performing motorcycle scheduled maintenance activities.

  - Perform motorcycle functionality check before service

    - Refer to report from the SA

3.4 Perform engine oil and oil filter changing

    - Determine types and functions of engine oil according to service manual

    - Select suitable oil  filter for motorcycle

    - Change engine oil and oil filter according to procedure and service manual

- Change drain plug washer

- Change  O-ring

    - Store used engine oil in drum approved from DOE (Department of Environmental)

3.5. Perform final drive system servicing  according to service manual

    - Identify  types of final drive system  to service

- chain

- belt

- shaft

- gear

    - Service final drive system according to service manual

    - Select suitable final drive gear oil for motorcycle for belt, shaft and gear system

    - Change final drive gear oil according to procedure and service manual

        - Change drain plug washer

    - Chain lubricant and freeplay adjustment

    - Store used engine oil in drum approved from DOE

  - Perform air filter changing

    - Determine types  and functions of air filter

- Refer to service job sheet and service manual

    - Check air filter according to procedure and service manual

- Check condition of air filter such as

- colour changing

- oil

- wet/moist

- etc

- Check mileage or duration of motorcycle.

  - Perform spark plugs changing

    - Determine types and functions of spark plugs. Refer to service job sheet and service manual

    - Change spark plugs according to procedure and service manual

  - Perform coolant changing

    - Select suitable coolant  for motorcycle

    - Check coolant level according to procedure and service manual

- Refill coolant  if necessary

- Change coolant according to duration

  - Perform valve clearance adjustment

    - Determine types and functions of valve clearance component. Refer to service job sheet and service manual

    - Check valve clearance according to procedure and service manual . To perform this step, you must alert to  condition below :

- engine noise

- engine loose power due to valve clearance

- duration

- poor engine idling

    - Adjust  valve clearance according to procedure and service manual

- Interpret adjustment table for shim type

  - Perform carburetor tuning

    - Check air fuel ratio according to procedure and service manual

- You can observe the change of plug color, engine idling and smell to detect the  in-equivalent ratio.

  - Perform screw and nut tightening according to service manual

  - Perform motorcycle functionality check after service

    - Reset meter system

    - Check suspension system, electrical system, brake system, drive system, wheel system, fuel system and engine system according to manufacturer’s specification

    - Inform SA that motorcycle ready for a test ride

    - Carry out a test ride on serviced motorcycle

  - Record scheduled maintenance servicing activities

  - Submit record to SA

  - Carry out housekeeping activities such as

    - 5’s

  - Comply with attitude, safety and environment listed below when performing this activity

| Attitude | - Peruse the manufacturer’s specification and maintenance manual - Adhere to workshop safety practice - Peruse the manufacturer’s specification and maintenance manual - Adhere to DOE requirements - Meticulous in preparing report |
| --- | --- |
| Safety | - Wear appropriate PPE - Handle wiring tools, equipment and testing tools with care - Ensure tools and equipment in good working condition - Wear appropriate PPE |
| Environment | i. Store used engine oil in drum approved from DOE |

  - Apply core abilities listed below when performing this activity

| Social Skills | Core Abilities |
| --- | --- |
| Communication skills | 02.03 Communicate clearly |
| Conceptual skills | 01.01 Identify and gather information 02.01 Interpret and follow manuals, instructions and SOP’s |
| Interpersonal skills | 03.06 Respond appropriately to people and situations |
| Multitasking and prioritizing | 01.02 Document information, procedures or processes 02.04 Prepare brief reports and checklists using standard form |
| Self-discipline | 03.03 Accept responsibility for own work and work area 03.02 Demonstrate integrity and apply ethical practices |
| Teamwork | 03.01 Apply cultural requirements for the workplace |
| Learning skill | 06.01 Understand systems |
| Leadership skill | 03.06 Respond appropriately to people and situations 03.03 Accept responsibility for own work and work area |

- EVALUATING

You are required to evaluate motorcycle scheduled maintenance activities using checklist below.

| A | ASSESSMENT CRITERIA (60%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Engine oil and oil filter changed according to procedure |  |  |  |  |  |  |  |  |  |  |
| 2. | Air filter changed according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 3. | Final drive gear oil changed according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 4. | Spark plugs changed according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 5. | Coolant changed according to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 6. | Valve clearance adjusted to procedure and manufacturer’s manual |  |  |  |  |  |  |  |  |  |  |
| 7. | Check suspension system, electrical system, brake system, drive system, tire pressure, fuel system and engine system according to manufacturer’s specification |  |  |  |  |  |  |  |  |  |  |
|  | SUBTOTAL | A1 |  |  |  |  | A2 |  |  |  |  |
|  | FULL MARKS | 49 |  |  |  |  | 49 |  |  |  |  |
| B | ATTITUDE/SAFETY/ ENVIRONMENT (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Attitude |  |  |  |  |  |  |  |  |  |  |
| 2. | Safety |  |  |  |  |  |  |  |  |  |  |
| 3 | Environmental |  |  |  |  |  |  |  |  |  |  |
|  |  | B1 |  |  |  |  | B2 |  |  |  |  |
|  |  | 21 |  |  |  |  | 21 |  |  |  |  |
| C | EMPLOYABILITY SKILLS (SOCIAL SKILLS) (20%) | MARKS GIVEN BY APPRENTICE |  |  |  |  | MARKS GIVEN BY COACH |  |  |  |  |
|  |  | 0 | 1-2 | 3-4 | 5-6 | 7 | 0 | 1-2 | 3-4 | 5-6 | 7 |
| 1 | Communication skills |  |  |  |  |  |  |  |  |  |  |
| 2 | Conceptual skills |  |  |  |  |  |  |  |  |  |  |
| 3 | Interpersonal skills |  |  |  |  |  |  |  |  |  |  |
| 4 | Multitasking and prioritizing |  |  |  |  |  |  |  |  |  |  |
| 5 | Self-discipline |  |  |  |  |  |  |  |  |  |  |
| 6 | Teamwork |  |  |  |  |  |  |  |  |  |  |
| 7 | Learning skill |  |  |  |  |  |  |  |  |  |  |
| 8 | Leadership skill |  |  |  |  |  |  |  |  |  |  |
|  |  | C1 |  |  |  |  | C2 |  |  |  |  |
|  | FULL MARKS | 56 |  |  |  |  | 56 |  |  |  |  |

CALCULATION TABLE

|  | MARKS GIVEN BY APPRENTICE | MARKS GIVEN BY COACH | WEIGHTED MARKS GIVEN BY APPRENTICE | WEIGHTED MARKS GIVEN BY COACH |
| --- | --- | --- | --- | --- |
| ASSESSMENT CRITERIA | A1 | A2 | A1 /49 X 60 | A2 /49 X 60 |
| ATTITUDE,SAFETY & ENVIRONMENT | B1 | B2 | B1 /21X 20 | B2 /21 X 20 |
| EMPLOYABILITY SKILLS (SOCIAL SKILLS) | C1 | C2 | C1 /56X 20 | C2 /56 X 20 |
| Total |  |  | X | Y |
| Ratio of Percentage (Apprentice: Coach) |  |  | P% | Q% |
| Grand Total |  |  | (P/100 x X) + (Q/100 x Y) |  |

COMMENTS/ RECOMMENDATIONS BY COACH

_________________________			  ______________________

COACH: 					              APPRENTICE:

DATE:							  DATE: